This logic controls the leg system (both hip and knee joints) to execute a complete "squat-hold-stand" cycle. It utilizes linear interpolation to ensure smooth transitions between target angles, ultimately returning to and maintaining a stable upright posture after 16 seconds.

| Phase | Absolute Time (t) | Relative Time (tau) | Action Description | Hip Angle | Knee Angle |
| --- | --- | --- | --- | --- | --- |
| Initial Wait | 0s - 5s | - | Hold upright (forced lock at 0 if in Wheel Mode). | 0° | 0° |
| Squat Down | 5s - 8s | 0s - 3s | Joints bend linearly to lower the chassis. | 0° ➔ +35° | 0° ➔ -70° |
| Hold Squat | 8s - 13s | 3s - 8s | Maintain deep squat posture for 5 seconds to stabilize the body. | +35° | -70° |
| Stand Up | 13s - 16s | 8s - 11s | Joints return to neutral linearly, pushing the chassis back to its original height. | +35° ➔ 0° | -70° ➔ 0° |
| Hold Stand | > 16s | > 11s | Sequence complete; maintain a standing posture indefinitely. | 0° | 0° |